Electrical Setup

These are the components used.

Arduino 1 = Inputs + LCD + IR beacon detection

Arduino 2 = Line following + DC motors + shooter ESC + servo flap

LM2596 = 12V battery to regulated 5V supply

PCA9685_1=DC motors

PCA9685_2 = ESC + servo only

The high-level connections are given below.

12V Battery Positive

|

|---- Emergency stop switch ---- L298N +12V motor input

|

|------- ESC main power positive

|

|---- Buck Converter IN+

12V Battery Negative

|

|---- L298N GND

|---- ESC main power negative

|---- Buck Converter IN-

|---- Arduino 1 GND

|---- Arduino 2 GND

|---- PCA9685_1 GND

|---- PCA9685_2 GND

|---- LCD GND

|---- Sensors GND

Buck Converter:

| Pin | Connect To |
| --- | --- |
| IN+ | Emergency Switch Terminal |
| IN- | 12V battery negative |
| OUT+ | 5V rail |
| OUT- | GND rail |

Arduino 1:

| Arduino 1 Pin |  | Connect To |  | Purpose |
| --- | --- | --- | --- | --- |
| D2 |  | IR beacon sensor 1 OUT |  | Frequency detection |
| D3 |  | IR beacon sensor 2 OUT |  | Frequency detection |
| D4 |  | Start toggle switch |  | Start/game mode input |
| D5 |  | Team color toggle switch |  | Red/blue team selection |
| D8 |  | LCD blue backlight control pin |  | Changes LCD color |
| D10 |  | Arduino 2 D10 |  | Serial TX to Arduino 2 |
| A4 |  | LCD SDA |  | I2C data |
| A5 |  | LCD SCL |  | I2C clock |
| 5V |  | Buck converter 5V OUT+ |  | Power |
| GND |  | Common GND |  | Ground |
| Start/Stop Toggle Switch: Pin | Connect To |  |
| One side | Arduino 1 D4 |  |
| Other side | GND |  |

Team Color Toggle Switch:

| Pin | Connect To |
| --- | --- |
| One side | Arduino 1 D5 |
| Other side | GND |

| IR Sensor 1: Pin | Connect To |
| --- | --- |
| VCC | 5V |
| GND | GND |
| OUT / D3 | Arduino 1 D2 |

| IR Sensor 2: Pin | Connect To |
| --- | --- |
| VCC | 5V |
| GND | GND |
| OUT / D3 | Arduino 1 D3 |

LCD:

| Pin | Connect To |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | Arduino 1 SDA |
| SCL | Arduino 1 SCL |
| Pin 18 | Arduino 1 D8 |

| Arduino 1 | Arduino 2 | Purpose |
| --- | --- | --- |
| D10 | D10 | Arduino 1 TX → Arduino 2 RX |
| GND | GND | Common ground |

| Arduino 2 Pin | Connect To | Purpose |
| --- | --- | --- |
| D10 | Arduino 1 D10 | Serial RX from Arduino 1 |
| SDA | PCA9685 SDA + line follower SDA | I2C data |
| SCL | PCA9685 SCL + line follower SCL | I2C clock |
| 5V | Buck Converter 5V Out+ | Power |
| GND | Common GND | Ground |

| SparkFun Line Sensor: Pin | Connect To |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | Arduino 2 SDA |
| SCL | Arduino 2 SCL |

| L298N Pin | Connect To |
| --- | --- |
| ENA | PCA9685_1 Ch 5 |
| IN1 | PCA9685_1 Ch 0 |
| IN2 | PCA9685_1 Ch 1 |
| IN3 | PCA9685_1 Ch 2 |
| IN4 | PCA9685_1 Ch 3 |
| ENB | PCA9685_1 Ch 4 |
| GND | Common GND |
| +12V | +12V |

OUT1	                 Left motor pair terminal 1
OUT2	                 Left motor pair terminal 2
OUT3	                 Right motor pair terminal 1
OUT4	                 Right motor pair terminal 2

| PCA9685_1: Pin | Connect To |
| --- | --- |
| VCC | 5V from Buck Converter |
| GND | Common GND |
| SDA | Arduino 2 SDA |
| SCL | Arduino 2 SCL |
| V+ | 5V from Buck Converter |

| PCA9685_2: Pin | Connect To |
| --- | --- |
| VCC | 5V from Buck Converter |
| GND | Common GND |
| SDA | Arduino 2 SDA |
| SCL | Arduino 2 SCL |
| V+ | 5V from Buck Converter |
| Servo: Wire | Connect To |
| Signal | PCA9685_2 CH1 signal |
| Red | PCA9685_2 V+ |

Black	GND

| ESC : Wire | Connect To |
| --- | --- |
| Signal | PCA9685_2 CH0 signal |
| Ground | PCA9685_2 GND |

ESC power positive           12V battery positive

| ESC power negative | Battery negative |
| --- | --- |